ASSESSMENT OF CLEAN & SAFE CONDITIONS IN THE RESIDENCE

(Regulation 299/10 s. 26(1)(a))

Entrance to Building/Home

• Obstacles to entrance and exit (e.g., bushes, trees).

• Clutter, accumulated garbage

• Poorly illuminated entrance, steps and/or walkways.

• Lack of or inappropriate handrails on exterior stairways.

• Poor condition of exterior stairs (e.g., broken, steep, narrow).

• Steps do not allow secure footing (i.e., uneven and not of same size or height, cannot see edges of steps clearly).

• Ice/snow on sidewalk and/or driveway.

• Walkways uneven, pot-holed, or slippery.

• Presence of insects and/or rodents.

Chemical/Biological/Environmental Hazards

• Unsanitary conditions (e.g., accumulation of garbage, smell of urine).

• Presence of rodents and/or insects (i.e., potential sources of disease, allergic reactions, bites).

• Inadequate plumbing (e.g., cannot access bathroom facilities, drinking water concerns).

• Presence of noxious fumes (e.g., environmental tobacco smoke).

• Kitchen ventilation systems or range exhausts do not function properly causing indoor air pollutants.

Fall Hazards

• Lamp, extension and telephone cords placed in the flow of traffic.

• Slippery area rugs and runners.

• Floors are not clean or are slippery.

• Floors contain loose or broken tiles.

• Lack or inappropriate handrails on interior stairways (e.g., not secure, not long enough).

• Poor condition of stairs (e.g., broken, steep, narrow).

• Worn or torn coverings are on the steps.

• Steps do not allow secure footing (i.e., uneven and not of same size or height, cannot see edges of steps clearly).

• Clutter on stairways/hallways.

• Open stairway – fall risk (e.g., no railing).

Fire Hazards

• Electrical cords running beneath furniture and rugs or carpeting.

• Electrical cords attached to walls, baseboards, etc. with nails or staples.

• Frayed or cracked electrical cords.

• Excessive loads on extension cords.

• Extension cords and appliance cords located close to the sink or range areas.

• Towels, curtains and other flammables close to the range.

• Small electrical appliances plugged in when not in use.

• Exposed wiring/lack of cover plates on outlets/switches.

• Outlets and switches unusually warm or hot to the touch.

• Inappropriate size and type of light bulbs for fixtures.

• Heaters with three-hole plug not used in three-hole outlet.

• Small stoves and heaters in places where they could be knocked over and close to furnishings and flammable materials.

• Clogged chimneys.

• Fire sources (heaters, hot plates, teapots) located close to bed or bedding.

• Electric blankets tucked in/have things on them.

• Gasoline, paints, solvents and other products that give off vapors or fumes stored close to ignition sources.

• Containers of volatile liquids not tightly capped.

• Improperly located smoke detectors (i.e., smoke detectors should be near bedrooms either on ceiling or wall (6" to 12" below the ceiling) and accessible to facilitate replacement of batteries).

• Absence of or improperly placed/working smoke detectors and/or carbon monoxide detectors.

• No portable fire extinguisher.

• Only one exit from home.